T-600S

Drilling and Tapping Machining Center

Technical Agreement

(This figure is for reference only)

- Technical description.

- Equipment technical description

T-600S Drilling and Milling Machining Center, expand the processing field of drilling and milling machines to meet customers' processing needs for large-size parts. X, Y, Z three-axis screws all use ball screws, and the three-axis guide rails use linear guide rails; the three axes use absolute servo motors; the spindle adopts a direct-coupled transmission structure, with the highest standard configuration The rotation speed reaches 24,000 rpm; the cast iron adopts Meehanite casting process and is completely heat treated to eliminate internal stress. The machine tool has high stability and reliability, integrating drilling, tapping, milling and other processing. It is widely used for small plate parts and disc-shaped parts in general industries, automotive parts, small mold processing, medical equipment and other industries. , Shell processing.

- Based on accumulated analytical technical data, the six-point support of the base provides high mechanical rigidity and ensures good and stable processing;

- The spindle motor and spindle are directly driven to reduce errors in high-speed tapping, and the high output force characteristics can be greatly utilized in high-speed and efficient processing;

- The high-speed spindle responds quickly and uses ceramic ball bearings, which greatly extends the service life of the spindle;

- Z-axis optimization improves stiffness and greatly increases Z-axis acceleration, reducing idle travel time;

- The machine tool has a large inclined chip removal angle, the machine tool removes chips quickly, and accelerates the return speed of the cutting fluid;

- High-speed servo tool change, PLC tool change optimal optimization, shorten tool change time, fast and stable, reduce non-processing time;

- With high-speed rigid tapping;

- By loading a high-speed PLC engine, the response speed is improved, the cycle is shortened, the processing detail parameters are optimized, the smoothness of the feed is improved, and the processing effect is improved;

- The machine design is simple and saves space. The front-end recessed design makes it easier for the operator to access the machine tool table.

- It adopts Mitsubishi M80VA system, with friendly human-machine interface, simple operation and high production performance.

- Working conditions

(1) Power supply: Three-phase AC, 380V ± 5%, 50Hz ± 1Hz. Main power cable and grounding wire: 6 mm² (for lengths up to 10 m) / 10 mm² (for lengths above 10 m).

(2) Ambient temperature: The ambient temperature should be maintained between 17°C and 25°C. If the machining tolerance is not stringent, the temperature can be relaxed to 5°C to 40°C. Storage and transportation temperature range: -20°C to 60°C.

(3) Room temperature fluctuation: The temperature fluctuation in the surrounding area should be within ±2°C over a 24-hour period.

(4) Relative humidity: Less than 75% (without condensation).

(5) Altitude: Below 1000 meters.

(6) Vibration: Below 0.2G;

(7) Foundation requirements: Load-bearing capacity of 32 kN/m² or higher.

(8) Keep away from light sources, vibration sources, and heat sources. Avoid proximity to high-frequency motors, discharge motors, welding machines, etc., to prevent electrical interference that could cause faults in the CNC system.

- Equipment parameters and configuration

2.1	Basic parameters

| Item |  |  | Unit | Parameter | Remark |
| --- | --- | --- | --- | --- | --- |
| Travel | Table travel (X axis) |  | mm | 600 |  |
|  | Saddle travel (Y axis) |  | mm | 400 |  |
|  | Spindle box travel (Z axis) |  | mm | 320 |  |
|  | Distance from spindle end face to worktable surface |  | mm | 140-460 |  |
|  | Distance from spindle center to column guide |  | mm | 445 |  |
| Worktable | Table size |  | mm | 700*400 |  |
|  | Max. load |  | kg | 250 |  |
|  | T-shaped slot (number of slots-slot width*spacing) |  | mm | 3-18*125 |  |
| Spindle | Motor power (rated/short time) |  | kW | 3.7/5.5 |  |
|  | Spindle torque (rated/short time) |  | N.m | 14.1/21 |  |
|  | Spindle speed |  | rpm | 20-24000 |  |
|  | Spindle diameter |  | mm | Ф100 |  |
|  | Handle Specifications |  | / | BT30 |  |
|  | Pull stud specification |  | / | P40T-I（MAS403） |  |
| Guide | X-axis |  | / | 25 |  |
|  | Y axis |  | / | 30 |  |
|  | Z axis |  | / | 35 |  |
| Drive | Lead screw X/Y/Z |  | mm | X: 28 Y: 28 Z: 32 |  |
|  | Servo motor power X/Y/Z |  | kW | 1.5/1.5/2.2 |  |
| Speed | Cutting feed speed range |  | mm/min | 1-15000 |  |
|  | X, Y, Z axis rapid speed |  | m/min | 48/48/48 |  |
| Precision | Positioning accuracy (X/Y/Z) |  | mm | 0.006 | GB/T 18400.4 (Full travel) |
|  | Repeatability(X/Y/Z) |  | mm | 0.004 |  |
| Tool magazine | Tool magazine capacity |  | T | 21 |  |
|  | Max. tool weight |  | kg | 3 |  |
|  | Max. tool length |  | mm | 300 |  |
|  | Max. tool diameter (full /adjacent empty) |  | mm | Ф60/Ф80 |  |
| Other | CNC system |  |  | Mitsubishi M80VA |  |
|  | Air supply | Flow | L/min | ≥200（ANR） |  |
|  |  | Air pressure | MPa | 0.6-0.8 |  |
|  | Total power capacity |  | kVA | 15 |  |
|  | Coolant tank volume |  | L | 145 |  |
|  | Machine dimensions |  | mm | 1880*2480*2330 | Corresponding to X*Y*Z axis |
|  | Machine weight (approx.) |  | kg | 2830 |  |

#: The dimensions of the machine are subject to change without prior notice due to configuration.

2.2	Customer-choosing configuration

| NO. | Item | Quantity | Remark |
| --- | --- | --- | --- |
| 1 |  |  |  |
| 2 |  |  |  |
| 3 |  |  |  |

# 2.3	Equipment standard configuration

| NO. | Item | Quantity | Remark |
| --- | --- | --- | --- |
| 1 | Direct-connected spindle 24000rpm | 1 |  |
| 2 | Spindle motor 20000rpm | 1 |  |
| 3 | 21T servo tool magazine | 1 |  |
| 4 | Centralized automatic lubrication system | 1 |  |
| 5 | Explosion-proof work light | 1 |  |
| 6 | Three color warning light | 1 |  |
| 7 | SD card | 1 |  |
| 8 | Fully sealed outer cover sheet metal protection (with top cover) | 1 |  |
| 9 | Air system | 1 |  |
| 10 | Standard technical documents | 1 |  |
| 11 | Mitsubishi M80VA system | 1 |  |
| 12 | Domestic tool magazine motor/ drive | 1 |  |
| 13 | Pneumatic clamp solenoid valve | 2 |  |
| 14 | Inter lock | 1 | Optional front automatic door, safety door lock cancelled |

- Brands of main parts of equipment

| Main parts |  |  |  |
| --- | --- | --- | --- |
| No. | Part name | Brand | Remark |
| 1 | Screw bearing | FAG/NSK | Germany/Japan |
| 2 | Ball screw | PMI |  |
| 3 | Spindle unit | Taikan(24000RPM) |  |
| 4 | Tool magazine | Taikan |  |
| 5 | Main pneumatic components | SMC/CKD |  |
| 6 | Spindle servo motor | Mitsubishi (24000RPM) | Japan |
| 7 | Three-axis drive servo motor | Mitsubishi | Japan |
| 8 | Controller system | Mitsubishi M80VA | Japan |
| 9 | Tool magazine drive servo motor | / | Domestic |
| 10 | Lubrication device | BAOTN |  |
| Note: In case of supply shortage, products of other brands will be used instead, but the quality and grade are not lower than the listed configuration. |  |  |  |

- Main Function List of the System (Some functions require specific host configurations to be implemented).

| No. | Function | Description | Notes |
| --- | --- | --- | --- |
| 1 | Maximum total control axis (servo axis + spindle + PLC axis) | 11 |  |
|  | Maximum control servo axes | 9 |  |
|  | Maximum control spindles | 4 |  |
| 2 | Simultaneous control axes | Max. 4 axes |  |
| 3 | Display unit | 8.4" color LCD screen |  |
| 4 | Program capacity | 2500KB (NC storage + NC storage 2) |  |
| 5 | High-speed machining mode |  |  |
| 6 | High-speed, high-precision control | Max 202 kBPM |  |
| 7 | SSS control |  |  |
| 8 | Tolerance control |  |  |
| 9 | Minimum instruction unit to 0.0001 mm | System can set minimum instruction unit |  |
| 10 | Imperial/metric conversion | Conversion of machine units |  |
| 11 | Decimal input | Programming instruction data specification method |  |
| 12 | Positioning |  |  |
| 13 | Linear interpolation | Multi-axis or single-axis coordinated movement control function |  |
| 14 | Circular interpolation | Circular machining function |  |
| 15 | Spiral interpolation | Spiral machining function |  |
| 16 | Polar coordinate interpolation |  |  |
| 17 | Polar coordinates |  |  |
| 18 | Automatic acceleration/deceleration control |  |  |
| 19 | High-speed rigid tapping | Reduces rigid tapping time and errors |  |
| 20 | Thread time constant switching |  |  |
| 21 | Manual rapid feed |  |  |
| 22 | JOG feed |  |  |
| 23 | Incremental feed |  |  |
| 24 | Handwheel feed |  |  |
| 25 | Pause |  |  |
| 26 | Background editing | Program editing while machining |  |
| 27 | Calculation input |  |  |
| 28 | Absolute, incremental setting | Programming instruction data specification method |  |
| 29 | Screensaver |  |  |
| 30 | Parameter guide |  |  |
| 31 | Alarm guide |  |  |
| 32 | Screenshot |  |  |
| 33 | Status, clock, operation, edit, maintenance display |  |  |
| 34 | Fault diagnosis | Check current machine fault status |  |
| 35 | Multi-language display function | Multi-language switching display |  |
| 36 | Input/output of machining programs, parameters, tool offsets, etc. |  |  |
| 37 | Multiple connection interfaces, network port, USB transfer | USB (max 32GB) |  |
| 38 | Digital, analog spindle available |  |  |
| 39 | Spindle positioning | Stops spindle at a fixed point for tapping, tool change, etc. |  |
| 40 | Spindle synchronization control |  |  |
| 41 | Tool compensation | 400 groups |  |
| 42 | Coordinate system | Basic: G52-G59, extended 48 coordinates |  |
| 43 | Coordinate rotation | Rotate orthogonal coordinates for easier programming |  |
| 44 | Plane selection | Use G17, G18, G19 commands to select the desired plane |  |
| 45 | Reference point 1, 2, 3, 4 return |  |  |
| 46 | Program segment skip selection | Skip program segments for more flexible operations |  |
| 47 | Single program segment | Program can execute one segment at a time |  |
| 48 | Dry run |  |  |
| 49 | Mechanical lock |  |  |
| 50 | Auxiliary function lock |  |  |
| 51 | Graphic inspection |  |  |
| 52 | Program restart | Restart after temporary interruption |  |
| 53 | Manual insert, MDI insert, etc. | Insert during operation |  |
| 54 | Rigid tapping retraction | Retraction during rigid tapping interruption |  |
| 55 | Macro programming | Variable programming available |  |
| 56 | Fixed cycle for hole processing | Fixed cycles for hole machining, e.g., deep-hole drilling |  |
| 57 | Set external input mirror | Achieve symmetrical machining via setting or external trigger |  |
| 58 | Arbitrary angle chamfering/corner R | Arbitrary angle chamfering/corner R |  |
| 59 | Drawing size input | Input dimensions as per drawing, e.g., corner chamfer |  |
| 60 | Programmable data input | Input of instruction program data such as parameters, tool offsets |  |
| 61 | Automatic corner scaling | Control for automatic corner machining scaling |  |
| 62 | Deceleration check | Check for deceleration stop position |  |
| 63 | Programming guide |  |  |
| 64 | Thread pitch, backlash compensation | Machine axis pitch and backlash compensation |  |
| 65 | Arc radius error compensation |  |  |
| 66 | Dynamic precision compensation | Smooth high-gain, dual feedback (closed-loop) |  |
| 67 | Tool life management 1 | Tool life monitoring |  |
| 68 | Tool life management 2 | Expanded tool life management |  |
| 69 | Tool life management quantity | 200 tools |  |
| 70 | Alarm stop for each machine group |  |  |
| 71 | Data protection for each user level |  |  |
| 72 | Stroke protection |  |  |
| 73 | Interlock |  |  |
| 74 | Operation history, data backup |  |  |
| 75 | Custom API library |  |  |
| 76 | MES interface library |  |  |
| 77 | Power consumption calculation |  |  |
| 78 | PLC position switch | PLC function, designed as needed |  |
| 79 | PLC servo off |  |  |
| 80 | PLC axis removal |  |  |
| 81 | Wireless network (Wi-Fi) |  |  |
| 82 | Cutting load control |  |  |
| 83 | Operation history display function |  |  |
| 84 | Expanded common variables III |  |  |
| 85 | PLC support for ST language |  |  |
| 86 | High-precision parameter adjustment function |  |  |
| 87 | OMR-CC |  |  |
| 88 | Drilling chip removal function |  |  |
| 89 | Fixed cutting reduction mode for drilling |  |  |
| 90 | Spline curve interpolation 2 (G61.4) | Supports parameter control |  |
| 91 | Tilted surface machining multiple instructions (G68.4) |  |  |
| 92 | Mechanical rotational center error measurement |  |  |
| 93 | Mechanical rotational center error compensation |  |  |
| 94 | QR code machining cycle |  |  |
| 95 | Laser machining control | Requires laser interface module |  |
| Special Note: For more functions, please refer to the M80V series specification selection guide. Due to differences in machine configurations and designs, please refer to the agreement for ordering. |  |  |  |

- Recommended Oils and Grease Chart (Refer to this chart according to the actual configuration of the equipment).

| Lubricating Part |  | Quantity | Recommended Oil |  | Remarks |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Room temperature 10-40℃ | Room temperature 5-10℃ |  |
| Guide rail screw | Oil lubrication | 2-4 L | Guide rail oil 68# | Guide rail oil 32# | 1. Please add oil when checking before daily work or when the warning appears. 2. Pay attention to changing oil at low temperature. 3. When using canned grease, the brand of canned grease must be consistent with the manufacturer of the oiler. |
|  | Oil lubrication | 0.7-0.8L | Lithium base grease 000# |  |  |
| Spindle screw cooler | Coolant cooler | 15-35L | BASF G48 antifreeze (no addition of any medium is allowed)） |  | 1. Please add antifreeze or oil when checking before daily work or when the warning appears. 2. Please change the oil at low temperature. |
|  | Oil Cooler | 20-40L | No. 10 Great Wall Spindle Oil | No. 5 Great Wall Spindle Oil |  |
| Motorized spindle | Oil and air lubrication | 2-4L | ENEOS SUPER MULPUS DX32 (Brand: VG32) |  | Please refuel during daily pre-work inspection or when a warning message appears. |
| Hydraulic station |  | 25-40L | Hydraulic oil HV32# | Hydraulic oil HV20# | 1. Check before work every day and replace with new oil every year. 2. Pay attention to replacing oil at low temperatures. |
| Booster cylinder |  | 0.1-0.2L | Hydraulic oil 32# | Hydraulic oil 20# | 1. Check before work every day, and add oil when the oil cup is only 1/3 full. 2. Pay attention to changing oil at low temperature |
| Cutting fluid |  | 100-400L | According to the workpiece material and processing conditions, select the appropriate cutting fluid (based on the actual tank capacity) |  | When the room temperature is below 5℃, please check whether the fuel tank is frozen before starting the machine. |
| Fourth axis |  | 0.8-1L | Gear Oil 150# | Gear Oil 100# | 1. Check before work every day and replace the oil every six months. 2. Pay attention to replace the oil at low temperatures. |
| Tool changer arm system |  | 7 L | Gear Oil 100# | 80W-90 Antifreeze diesel engine oil | 1. Check before work every day, and replace the oil every 2400 working hours or every six months. 2. Pay attention to replace the oil at low temperatures. |
| Requirements: |  |  |  |  |  |
| A. The coolant must be water-soluble; B. The coolant must be chemically stable and have lubricating properties; C. Ensure that the coolant is not toxic to the human body; D. Ensure that the coolant does not harden or swell items made of rubber and resin; E. Use low-foam coolant; F. If cutting fluid (oil) is not used for a long time, the protective cover needs to be maintained regularly. |  |  |  |  |  |

- Others

1. Matters not covered in this agreement shall be resolved through negotiation between the two parties.

2. This agreement is an attachment to the contract and will take effect simultaneously with the contract after being signed by Party A and Party B.

3. This agreement is made in duplicate, with Party A and Party B each holding one copy.