G-V1220A Double Column

Machining Center

Technology Agreement

| Party A（Buyer）： | Represent: Date: |
| --- | --- |
| Party B（Seller）：Shenzhen Create Century Machinery Co., Ltd. | Represent: Date: |

1、Equipment technical description, parameters and configuration

1、Equipment technical description

G-V1220A double column machining center is a large and medium-sized machine tool. The workpiece can automatically and continuously complete milling, drilling, boring, expanding, reaming, countersinking, tapping and other processes after one clamping. The machine tool is suitable for large Processing of various types of small and medium-sized batches of complex parts such as medium-sized boxes, plates, disks, valves, and shells.

- The whole machine structure is a double column structure, with integrated columns and beams designed to improve the overall rigidity of the columns. The basic large parts are made of high-quality resin sand molding and high-strength cast iron materials to ensure high rigidity and stable accuracy of the machine tool. The main castings have undergone finite element analysis, and the ribs are reasonably arranged to fully meet the needs of high-torque cutting of machine tools.

- The spindle adopts a high-speed spindle unit with a standard belt-driven BT50-6000rpm spindle, which has high cutting efficiency.

- The front end of the spindle adopts a labyrinth design, and the spindle has an air curtain protection function to prevent cutting fluid from entering the spindle bearing.

- The spindle has a central air blowing function and is equipped with an automatic circulating water coolant cooling device to reduce the temperature rise and thermal deformation of the spindle and maintain high machining accuracy.

- The main spindle box movement (Z-axis) is equipped with a nitrogen balance device. Even when moving at high speed, the counterweight will not shake; the precise ratio of the counterweight to the main spindle box weight can obtain the best processing characteristics, and the Z-axis drive motor has Good load characteristics.

- The three-axis feed adopts a direct connection drive method between the motor and the high-precision ball screw. The large-diameter ball screw is pre-stretched and installed to further improve the transmission rigidity and positioning accuracy.

- The X/Y-axis guideways adopt MSR45LE and MSR55LS roller linear rails respectively. The Z-axis guideways adopts MSR45LS roller linear rails, which have high rigidity, low friction, low noise, strong shock resistance, and can carry greater cutting forces.

- The guidewayss and ball screws of the machine tool adopt a timed and quantitative lubrication system, which can adjust the oil supply cycle and amount of each lubrication point, and realize fully automatic centralized lubrication through electrical system control.

- The chip removal of the machine tool adopts double screw conveyor, which is automatic, user-friendly and high-efficiency. It is also equipped with a front chain plate chip conveyor, which greatly reduces manual cleaning time.

- The machine tool water tank adopts a multi-layer filtration system structure to provide good cutting fluid for workpiece processing.

- working conditions

- Power supply: three-phase AC 380V±10%; 50Hz±1Hz; the machine tool should have reliable grounding: the grounding wire is a copper wire ≥16mm²; the grounding resistance is <4Ω.

- In order to keep the static accuracy of the machine tool within the guaranteed value range, the machine tool should be installed in an area not affected by air flow. It is necessary to keep the surrounding temperature between 17℃~25℃, the humidity between 40% and 75%, and The ambient temperature change within 24 hours should be within ±2°C, and the ambient temperature change from the ground to about 5 meters high should be maintained within 2°C.

- If the requirements for the parts being processed are not high, the ambient temperature range can be relaxed to 10°C ~ 40°C.

- Keep away from light sources, vibration sources and heat sources, and away from high-frequency generators, discharge motors, welding machines, etc. to avoid electrical interference that may cause the machine tool NC system to malfunction.

- If the voltage in the area where it is used is unstable, the machine tool should be equipped with a regulated power supply to ensure the normal operation of the machine tool.

- In order to ensure the normal operation of the equipment, if the compressed air source does not meet the cleanliness requirements, an air source purification device (dehumidification, oil removal, filtration) should be added before the air intake of the machine tool.

- In order to maintain the accuracy and stability of the machine tool, the foundation must be made strictly in accordance with the requirements of the foundation diagram provided by the company.

2. Equipment parameters and configuration

2.1. Basic parameters

| Item |  |  | Unit | Parameters | Remark |
| --- | --- | --- | --- | --- | --- |
| Processing range | X axis |  | mm | 2150 |  |
|  | Y axis |  | mm | 1500（+25 tool changing stroke） |  |
|  | Z axis |  | mm | 800 |  |
|  | Door width |  | mm | 1560 |  |
|  | Distance from spindle end face to worktable |  | mm | 100～900 |  |
|  | Distance from spindle center to column guide surface |  | mm | 474 |  |
| Worktable | Size |  | mm | 2000*1250 |  |
|  | Max load |  | kg | 4000 |  |
|  | T-shaped slot (number of slots-slot width*spacing) |  | mm | 7-22*160 |  |
| Spindle | Motor power (rated/short time) |  | kW | 15/18.5 |  |
|  | Spindle torque (rated/short time) |  | N▪m | 191/315 |  |
|  | Spindle speed |  | r/min | 50～6000 |  |
|  | Spindle diameter |  | mm | Φ210 |  |
|  | Spindle taper |  |  | BT50 |  |
|  | Pull stud |  |  | P50T-I（MAS403） |  |
| Guideways | X-axis |  | mm | 2-45 Roller |  |
|  | Y axis |  | mm | 2-55 Roller |  |
|  | Z axis |  | mm | 2-45 Roller |  |
| Drive | Lead screw X/Y/Z |  | mm | 50*12/50*12/50*12 |  |
|  | Servo motor power X/Y/Z |  | kW | 3/3/3 |  |
| Speed | Cutting feed speed range |  | mm/min | 1-10000 |  |
|  | X, Y, Z axis rapid movement speed |  | m/min | 16/16/12 |  |
| Precision | Positioning accuracy（X/Y/Z） |  | mm | 0.015/0.012/0.012 |  |
|  | Repeat positioning accuracy（X/Y/Z） |  | mm | 0.008/0.008/0.008 |  |
| Tool magazine (optional) | Tool magazine capacity |  | 把 | 24 |  |
|  | Maximum tool weight |  | kg | 18 |  |
|  | Tool length |  | mm | 350 |  |
|  | Maximum diameter (full tool/adjacent empty tool) |  | mm | Φ110/Φ200 |  |
| Other | Z-axis counterweight |  |  | Nitrogen balance |  |
|  | CNC system |  |  | FANUC 0i-MF PULS (5)/β motor |  |
|  | Air | Flow | L/min | 280（ANR） |  |
|  |  | Air pressure | MPa | 0.5~0.8 |  |
|  | Total power capacity |  | KVA | 40 |  |
|  | Cooling box volume |  | L | 400 |  |
|  | Machine tool appearance dimensions |  | mm | 5000*4500*4000 | Corresponds to X*Y*Z axis |

2.2. Equipment standard configuration

| NO. | Item | Quantity | Remark |
| --- | --- | --- | --- |
| 1 | FANUC 0i-MF PULS (5)/β motor CNC system | 1 |  |
| 2 | Operation panel | 1 |  |
| 3 | Pneumatic system | 1 |  |
| 4 | Air gun | 1 |  |
| 5 | Automatic lubrication system | 1 |  |
| 6 | Spindle cooling system (water cooling) | 1 |  |
| 7 | Workpiece cooling system | 1 |  |
| 8 | Spiral chip conveyor | 2 |  |
| 9 | Full protection | 1 | No cap |
| 10 | Three-color signal light | 1 |  |
| 11 | Lighting device | 1 |  |
| 12 | Basic installation kit | 1 |  |
| 13 | Operating foot pedal | 1 |  |
| 14 | Technical documents | 1 |  |
| 15 | Chain plate chip conveyor | 1 |  |

2.3、Customer chooses configuration

| NO. | Item | Whether to Choose | Remark |
| --- | --- | --- | --- |
| 1 | Column height increase 200mm | □ | The distance between the spindle end face and the work surface is 300-1100 mm |
| 2 | BT50-24T disc type tool magazine | □ | TaiKan |
| 3 | 2MPa center through coolant | □ |  |
| 4 | 3MPa center through coolant | □ |  |
| 5 | Coolant gun | □ |  |
| 6 | BT50 manual right-angle milling head (2000 r/min) manual head change/ release/ rotation | □ | Cannot process inclined surfaces |
| 7 | grating ruler | □ | Fagor |
| 8 | grating ruler | □ | Heidenhain |
| Note: If you select this configuration, please mark √ in the corresponding item box. This table only lists the main selected configurations. For other selected configuration items, please consult the manufacturer and fill in the blanks. |  |  |  |

- 2.4 Brands of main parts of equipment

| NO. | Item | Brand | Precision level |
| --- | --- | --- | --- |
| 1 | Control system | FANUC 0i-MF PULS（5） | High performance level |
| 2 | Spindle unit | TaiKan | High precision grade |
| 3 | Spindle bearings | Japan NSK/Germany FAG | P4 level |
| 4 | Three-axis bearings | Japan NSK/Germany FAG | P4 level |
| 5 | Three-axis ballscrew | Taiwan PMI/HIWIN |  |
| 6 | Three-axis guideways | Taiwan PMI/HIWIN /REXROTH/YINA | P（Precision grade） |
| 7 | Automatic lubrication pump | Domestic |  |
| 8 | server driver | Japan FANUC |  |
| 9 | Main electrical components | French Schneider |  |
| 10 | Main pneumatic components | Japan SMC |  |

- Note: The manufacturer reserves the right to replace the product with the same brand

- 3、FANUC 0i-MF PULS（5）System main function table (some functions need to be combined with the configuration of the host to be realized)

| Serial number | Function | Illustrate |
| --- | --- | --- |
| 1 | Number of control paths | 1 |
| 2 | Maximum total control axis (servo axis + spindle) | 6 |
|  | Maximum number of control servo axes | 5 |
|  | Maximum number of control spindles | 1 |
| 3 | Simultaneous Control Axis | 4 |
| 4 | Display unit | 10.4" color LED |
| 5 | Servo HRV control | High-speed, high-precision servo control |
| 6 | Program capacity | 2M |
| 7 | I/O Link i | 1024/1024 |
| 8 | Automatic data backup | Backup of machine data |
| 9 | Memory Card Edit/Run | DNC online processing |
| 10 | Program folder management | Functions of using macro programs and machining programs |
| 11 | FSSB high speed rigid tapping | Reduce rigid tapping time and errors |
| 12 | Intelligent backlash compensation | Backlash compensation |
| 13 | PMC axis control | Used when the tool magazine is a servo tool magazine |
| 14 | Data transfer via embedded Ethernet | Data transfer via embedded Ethernet |
| 15 | USB transfer program to CF card | Data transfer function |
| 16 | Srevo Guide Mate | Servo optimization |
| 17 | Spindle smart load meter | Monitor status such as spindle current |
| 18 | AI contour control | Optimum speed and acceleration control for block read-ahead |
| 19 | Smooth tolerance control | Determine the appropriate machining speed by commanding allowable tolerances |
| 20 | Smart overlap | Achieve shortening of component machining cycle time |
| 21 | Spindle HRV control | High-speed, high-precision servo control |
| 22 | Imperial/Metric conversion | Conversion of machine tool units |
| 23 | HRV2 control | High-speed, high-precision servo control |
| 24 | HRV3 control | High-speed, high-precision servo control |
| 25 | FANUC SERVO GUIDE | Servo adjustment for high-speed, high-precision processing in a short time |
| 26 | Flexible feed gear | Any DMR |
| 27 | Minimum setting unit (0.0001mm, 0.0001deg, 0.00001 inch) | The smallest unit of the system |
| 28 | Mirror | Each axis |
| 29 | Rigid tapping retraction | Rigid tapping retraction |
| 30 | Buffer Register | The system can cache data |
| 31 | Positioning | Spindle positioning function for tapping, tool change |
| 32 | Straight line interpolation | Linear machining function |
| 33 | Arc interpolation | Arc processing function |
| 34 | Spiral interpolation | Spiral processing function |
| 35 | Thread cutting, synchronous feed | Thread cutting, synchronous feed |
| 36 | Flip of the axis of rotation | Flip of the axis of rotation |
| 37 | Any angle chamfer/corner R | Any angle chamfer/corner R |
| 38 | Fixed cycle for drilling | Fixed cycle for drilling |
| 39 | No stop reference point setting | No stop reference point setting |
| 40 | Automatic corner magnification | Automatic corner machining override control |
| 41 | Scaling | Scaling |
| 42 | Plane selection | G17 G18 G19 |
| 43 | Programmable mirror | Programmable mirroring of each axis |
| 44 | Single program segment | The program can be executed in a single section |
| 45 | Program restarts | After the program is temporarily interrupted, it can be restarted |
| 46 | Tool diameter, tool tip radius compensation | Tool parameter compensation |
| 47 | Tool life management | Tool life management |
| 48 | Tool life management extension | Tool life management extension |
| 49 | Backlash compensation | Machine tool axis backlash compensation function |
| 50 | location tracking | location tracking |
| 51 | Abnormal load detection | Abnormal load detection on each axis |
| 52 | Position switch | Program protection function |
| 53 | Background editing | Program editing can be performed in edit mode |
| 54 | National language display function | System operation interface language display function 10 kinds |
| 55 | Self-diagnosis function | Can monitor machine tool status |
| 56 | Help function | See system alarm |
| 57 | Nanometer interpolation | Nanometer interpolation |
| 58 | Workpiece coordinate system | G52~G59 |
| 59 | Absolute/incremental instructions | In the same program section can be mixed |
| 60 | Fault diagnosis | Can find the current machine fault situation |
| 61 | Clear the screen | The screen can be cleared manually or automatically |
| 62 | Power consumption monitoring | Can check the machine power consumption |

- 4、Recommended oil and grease

| Lubrication part | Name | Capacity | Trademark | Manufacturer recommendation | Remark |
| --- | --- | --- | --- | --- | --- |
| hydraulic station | Hydraulic oil | 40L | No. 46 | Shell Tellus 46 Or Great Wall HF-2 46 | Replace it after 3 months of initial use; replace it every 6 months thereafter (optional) |
| Spindle water cooling machine | Antifreeze coolant | 15L |  | BASF (BASF): Gulishun G48; Mobil: -45℃ antifreeze coolant | Replace every 3 months |
| Lubrication pump | rail oil | 4L | 32#(winter) 68# (summer) | Shell Tonna T32 (<10 degrees) T68 (>10 degrees) Or the great wall HG32 (<10 degrees) HG68 (>10 degrees) | Refill when the oil level is below the minimum level line |

5、Others

- Matters not covered in this agreement shall be resolved through negotiation between the two parties.

- This agreement is an attachment to the contract and will take effect simultaneously with the contract after being signed by Party A and Party B.

- This agreement is made in duplicate, with Party A and Party B each holding one copy