G-V1520B Double Column

Machining Center

(Full protective sheet metal)

Technology Agreement

(This picture is for reference only)

| Party A（Buyer）： | Represent: Date: |
| --- | --- |
| Party B（Seller）：Shenzhen Create Century Machinery Co., Ltd. | Represent: Date: |

Directory

1. Technical description, parameters and configuration of equipment

2. Brands of main parts of equipment

3. System main function table

4. Recommended oil and grease table for equipment

5. Others

1、Equipment technical description, parameters and configuration

1、Equipment technical description

The G-V1520B double column machining center is mainly used to process small and medium-sized complex parts of ferrous and non-ferrous metals. It has the characteristics of high speed, high precision, high flexibility and environmental protection. Its performance indicators and accuracy indicators fully comply with national standards. The products are modularly designed and can be serialized and customized according to market demand. Its good performance-price ratio is the best choice for domestic and foreign customers.

The quality of the processed and assembled finished products of all machine tool parts complies with the product drawings and relevant technical requirements. Its safety standards comply with the relevant provisions of GB15760-2004 "General Technical Conditions for Safety Protection of Metal Cutting Machine Tools", and its accuracy standards comply with GB/T17421-2000 "Machine Tool Inspection" General Principles" and the relevant provisions of GB/T 25658.1-2010 "CNC Fixed Beam Gantry Boring and Milling Machine".

- The whole machine structure: the machine adopts the double column frame fixed, the worktable moving structure. The foundation large parts are made of high quality resin sand molding and high strength cast iron material, so that the machine can get high rigidity and stable accuracy. The main castings have been analyzed by finite element analysis, and the grid layout is reasonable, which fully meets the needs of high torque cutting of the machine tool.

- Spindle: The machine tool comes standard with a high-rigidity mechanical spindle that is directly connected to the variable speed gearbox. It has large low-speed torque, high-speed and high-precision, low noise and easy maintenance, and can meet the requirements of boring, milling and drilling. The standard water cooler cools the spindle, increases the service life of the spindle, and reduces the impact of thermal deformation of the spindle on machining accuracy.

- Guideways: The bed guideways (X-axis), beam guideways (Y-axis) and ram guideways (Z-axis) all adopt imported heavy-duty roller guideways, which have low friction, strong load-bearing capacity, low high-speed vibration, and no crawling at low speeds. High positioning accuracy. The crossbeam guideways adopts a stepped arrangement to increase the guide rail span and improve the load-bearing capacity. It has good load-bearing performance and ensures stable cutting during processing.

- Drive: The three feed axes of X, Y and Z are all driven by FANUC feed motors, which have good stability and reliable operation. The X-axis and Y-axis use large-lead ball screw drive structures; the Z-axis uses a motor directly connected to the ball screw, and the Z-axis uses a nitrogen balance cylinder to balance the weight of the ram.

- System: Equipped with a high-performance FANUC 0i-MF Plus (5) CNC system to ensure the stability of machine tool control and the CNC processing functions and auxiliary functions required by users.

- Working conditions:

- Power supply: three-phase AC 380V±10%; 50Hz±1Hz; the machine tool should have reliable grounding: the grounding wire is a copper wire ≥16mm²; the grounding resistance is <4Ω.

- In order to keep the static accuracy of the machine tool within the guaranteed value range, the machine tool should be installed in an area not affected by air flow. It is necessary to keep the surrounding temperature between 17℃~25℃, the humidity between 40% and 75%, and The ambient temperature change within 24 hours should be within ±2°C, and the ambient temperature change from the ground to about 5 meters high should be maintained within 2°C.

- If the requirements for the parts being processed are not high, the ambient temperature range can be relaxed to 10°C ~ 40°C.

- Keep away from light sources, vibration sources and heat sources, and away from high-frequency generators, discharge motors, welding machines, etc. to avoid electrical interference that may cause the machine tool NC system to malfunction.

- If the voltage in the area where it is used is unstable, the machine tool should be equipped with a regulated power supply to ensure the normal operation of the machine tool.

- In order to ensure the normal operation of the equipment, if the compressed air source does not meet the cleanliness requirements, an air source purification device (dehumidification, oil removal, filtration) should be added before the air intake of the machine tool.

- In order to maintain the accuracy and stability of the machine tool, the foundation must be made in strict accordance with the requirements of the foundation map provided by the company. When the user chooses to sink the machine tool, the user-made protective covers and support parts for the foundation anti-seismic ditch and sinking pit; our company provides the necessary technology support.

2. Equipment parameters

| Item |  |  | Unit | Parameters | Remark |
| --- | --- | --- | --- | --- | --- |
| Processing range | X axis |  | mm | 2200 |  |
|  | Y axis |  | mm | 2200 |  |
|  | Z axis |  | mm | 1000 |  |
|  | Distance from spindle end face to worktable |  | mm | 250～1250 |  |
|  | Effective door width |  | mm | 1950 |  |
| Worktable | Size |  | mm | 1500*2000 |  |
|  | Max load |  | Kg | 6000 |  |
|  | T-shaped slot |  | mm | 28 |  |
| Spindle | Spindle motor power (rated/maximum) |  | KW | 15/18.5 |  |
|  | Spindle speed ( gear drive) |  | r/min | 4000～6000(5500 when water is supplied from the optional center) | Within 120 minutes |
|  |  |  |  | 40～4000 | continuous |
|  | Output torque (rated/maximum) |  | Nm | 807/1331 |  |
|  | Spindle diameter |  | mm | φ210 |  |
|  | drive ratio |  |  | 1:1/1:5.64 |  |
|  | Ram section |  | mm | 420*420 |  |
|  | Spindle taper/Pull stud |  |  | BT50/ P50T-I（MAS403） |  |
| Speed | Cutting feed speed range |  | mm/min | 10000/10000/10000 |  |
|  | X, Y, Z axis rapid movement speed |  | mm/min | 15000/15000/15000 |  |
| Guideways | X/Y/Z |  |  | 2-45/2-55/2-55+2-45 |  |
| Ball screw | X/Y/Z（Diameter x Lead） |  | mm | 63x20/63x20/50x8 |  |
| Precision | Positioning accuracy（X/Y/Z） |  | No grating | 0.023/0.023/0.016 |  |
|  |  |  | With grating | 0.019/0.019/0.014 |  |
|  | Repeat positioning accuracy（X/Y/Z） |  | No grating | 0.014/0.014/0.010 |  |
|  |  |  | With grating | 0.012/0.012/0.009 |  |
| Other | Z-axis counterweight |  |  | Oil pressure + nitrogen balance |  |
|  | CNC system |  |  | FANUC 0I-MF PLUS(5)/βMotor |  |
|  | Air | Flow | L/min | 300 |  |
|  |  | Air pressure | MPa | 0.6~0.8 |  |
|  | Total power capacity |  | KVA | 50 |  |
|  | Cooling box volume |  | L | 500 |  |
|  | Machine tool appearance dimensions |  | mm | 6406*5576*5200 |  |
|  | Machine tool sheet metal protection |  | Set | Fully protective sheet metal | No cap |

- 3、Equipment standard configuration

| No. | Item | No. | Item |
| --- | --- | --- | --- |
| 1 | CNC System：FANUC 0i-MF PULS (5)/β motor | 13 | Hydraulic station |
| 2 | 10.4-inch monitor | 14 | Mechanical spindle |
| 3 | Handheld operating unit | 15 | Crossbeam guide rail protective cover |
| 4 | Hanging operation panel | 16 | Bed guide rail protective cover |
| 5 | Pneumatic system | 17 | Three-color signal light |
| 6 | Clean airgun | 18 | lighting device |
| 7 | Automatic lubrication system | 19 | Electric cabinet air conditioner |
| 8 | Spindle cooling system (water cooling) | 20 | Ram Nitrogen Balance System |
| 9 | Workpiece cooling system | 21 | Basic installation kit |
| 10 | Chain plate chip conveyor + chip collecting car | 22 | Technical documents |
| 11 | Spiral chip conveyor | 23 | Operating side pedals (not available for sunken foundations) |
| 12 | Gearbox cooling system (oil cooling) |  |  |

4、Customer chooses configuration

4.1 CNC System

| No. | Item | Whether to choose | Remark |
| --- | --- | --- | --- |
| 1 | FANUC 0I-MF PLUS (1) System | □ | High-end mold processing |
| 2 | (A02B-0304-R522; A02B-0304-J998 inclined surface processing and 3D hand wheel return function) | □ | If you select this option, you must also select the FANUC 0I-MF PLUS(1) system |

4.2 Other optional configurations

| No. | Item | Whether to choose | Remark |
| --- | --- | --- | --- |
| 1 | Sinking foundation | □ | The machine tool working table is 100 mm above the ground; the foundation anti-seismic ditch and sinking pit are made of user-made protective covers and supporting parts. |
| 2 | 24 disc tool magazine | □ | Tool magazine brand: TaiKan; maximum tool diameter (full tool/empty tool): Φ110mm/Φ200mm; maximum tool length: 350mm; maximum tool weight: 18KG, total tool weight: ≤360KG. |
| 3 | 40-tool chain tool magazine | □ | Tool magazine brand: Okada; maximum tool diameter (full tool/free space tool): Φ110mm/Φ200mm; maximum tool length: 350mm; maximum tool weight: 18KG, total tool weight: ≤600KG. |
| 4 | Grating ruler | □ | Fagor |
| 5 | Grating ruler | □ | Heidenhain |
| 6 | 2MPa center through coolant | □ | When selecting this option, the maximum spindle speed is 5500 |
| 7 | 3MPa center through coolant | □ | When selecting this option, the maximum spindle speed is 5500 |
| 8 | Manual right angle milling head BT50, manual head change, manual tool unclamping, manual rotation, 2000r/min; milling head uses tool diameter ≤φ125. (Installing a simple bracket for the workbench) | □ | If you need inclined surface processing, you must choose: (A02B-0304-R522; A02B-0304-J998 inclined surface processing and 3D handwheel return function) and FANUC 0I-MF PLUS (1) system |
| 9 | Column heightening 248 (whole column) | □ | The distance from the spindle end face to the work table changes to 500 to 1500, the height of the machine tool changes to 5450, and the rigidity is reduced by 20%. |
| 10 | Column heightening 248 (heightening pad) | □ | The distance from the spindle end face to the work table changes to 500 to 1500, the height of the machine tool changes to 5450, and the rigidity is reduced by 20%. |
| 11 | Chip removal water gun | □ |  |
| Note: If this configuration is selected, mark √ in the corresponding item box; the default is standard foundation, and the machine tool foot is level with the ground. |  |  |  |

2. Brands of main parts of equipment

| No. | Item | Brand | Remark |
| --- | --- | --- | --- |
| 1 | Controller system | FANUC | Japan |
| 2 | Spindle unit | TaiKan | Domestic |
| 3 | Ball screw | PMI/HIWIN | Taiwan |
| 4 | Screw bearing | NSK | Japan |
| 5 | Guideways | PMI/HIWIN /INA | Taiwan/Germany |
| 6 | Lubrication system | BAOTENG | Domestic |
| 7 | Main pneumatic components | SMC | Japan |
| 8 | Gearbox oil cooling machine | Tongfe | Domestic |
| 9 | Spindle water cooler machine | Saiyang | Domestic |
| 10 | Electrical cabinet air conditioner | Tongfe | Domestic |
| 11 | Main electrical components | Schneider | France |

- Note: The manufacturer reserves the right to replace the product with the same brand

- 3、Main system function list

- 1. FANUC 0I-MF PLUS (1) system main function table (some functions need to be combined with the configuration of the host to be realized)

| No. | Function | Illustrate | Remark |
| --- | --- | --- | --- |
| 1 | Number of control paths | 1 |  |
| 2 | Maximum total number of control axes (feed axis + spindle) | 6 |  |
|  | Maximum number of feed axes | 5 |  |
|  | Maximum number of spindles | 1 |  |
| 3 | Simultaneous Control Axis | 4 |  |
| 4 | Display unit | 10.4" color LED |  |
| 5 | Servo HRV control | High-speed, high-precision servo control |  |
| 6 | Program capacity | 2M |  |
| 7 | I/O Link i | 1024/1024 |  |
| 8 | Data backup | Backup of machine data |  |
| 9 | Memory Card Edit/Run | DNC online processing |  |
| 10 | Program folder management | Functions of using macro programs and machining programs |  |
| 11 | FSSB high speed rigid tapping | Reduce rigid tapping time and errors |  |
| 12 | Intelligent backlash compensation | Backlash compensation |  |
| 13 | Storage type pitch error compensation |  | Options |
| 14 | PMC axis control | Used when the tool magazine is a servo tool magazine |  |
| 15 | Data transfer via embedded Ethernet | Data transfer via embedded Ethernet |  |
| 16 | USB transfer program to CF card | Data transfer function |  |
| 17 | Srevo Guide Mate |  |  |
| 18 | Spindle smart load meter | Monitor status such as spindle current |  |
| 19 | HRV control | High-speed, high-precision servo control |  |
| 20 | Smart overlap | Achieve shortening of component machining cycle time |  |
| 21 | Pre-processing preparation support function |  | Options |
| 22 | Bell-shaped acceleration and deceleration before read-ahead interpolation |  |  |
| 23 | Imperial/Metric conversion | Conversion of machine tool units |  |
| 24 | HRV3 control | High-speed, high-precision servo control | Options |
| 25 | FANUC SERVO GUIDE | Servo adjustment for high-speed, high-precision processing in a short time |  |
| 26 | Flexible feed gear | Any DMR |  |
| 27 | Minimum setting unit (0.0001mm, 0.0001deg, 0.00001 inch) | The smallest unit of the system |  |
| 28 | Mirror | Each axis |  |
| 29 | Rigid tapping retraction | Rigid tapping retraction |  |
| 30 | Buffer Register | The system can cache data |  |
| 31 | Positioning |  |  |
| 32 | Straight line interpolation | Linear machining function |  |
| 33 | Arc interpolation | Arc processing function |  |
| 34 | Cylindrical interpolation | Cylindrical processing function | Options |
| 35 | Spiral interpolation | Spiral processing function |  |
| 36 | Thread cutting, synchronous feed | Thread cutting, synchronous feed | Options |
| 37 | Flip of the axis of rotation | Flip of the axis of rotation |  |
| 38 | Any angle chamfer/corner R | Any angle chamfer/corner R |  |
| 39 | Fixed cycle for drilling | Fixed cycle for drilling |  |
| 40 | No stop reference point setting | No stop reference point setting |  |
| 41 | Automatic corner magnification | Automatic corner machining override control |  |
| 42 | Scaling | Scaling |  |
| 43 | Plane selection | G17 G18 G19 |  |
| 44 | Programmable mirror | Programmable mirroring of each axis |  |
| 45 | Single program segment | The program can be executed in a single section |  |
| 46 | Small diameter deep hole drilling cycle | Small diameter deep hole drilling cycle | Options |
| 47 | Program restarts | After the program is temporarily interrupted, it can be restarted |  |
| 48 | Tool compensation | Tool parameter compensation |  |
| 49 | Tool life management | Tool life management |  |
| 50 | Tool life management extension | Tool life management extension |  |
| 51 | Spindle stops accurately |  |  |
| 52 | location tracking | location tracking |  |
| 53 | Abnormal load detection | Abnormal load detection on each axis |  |
| 54 | Position switch | Program protection function |  |
| 55 | Background editing | Program editing can be performed in edit mode |  |
| 56 | National language display function | System operation interface language display function 10 kinds |  |
| 57 | Self-diagnosis function | Can monitor machine tool status |  |
| 58 | Help function | See system alarm |  |
| 59 | Nanometer interpolation | Nanometer interpolation |  |
| 60 | Workpiece coordinate system | G52~G59 |  |
| 61 | Select block to skip | 9 | Options |
| 62 | Absolute/incremental instructions | In the same program section can be mixed |  |
| 63 | Fault diagnosis | Can find the current machine fault situation |  |
| 64 | Clear the screen | The screen can be cleared manually or automatically |  |
| 65 | Power consumption monitoring | Can check the machine power consumption |  |

- 2.FANUC 0I-MF PLUS (1) system main function table (some functions need to be combined with the configuration of the host to be realized)

| No. | Function | Illustrate | Remark |
| --- | --- | --- | --- |
| 1 | Number of control paths | 1 |  |
| 2 | Maximum total control axis (servo axis + spindle) | 9 |  |
|  | Maximum number of control servo axes | 7 |  |
|  | Maximum number of control spindles | 2 |  |
| 3 | Simultaneous Control Axis | 4 |  |
| 4 | Display unit | 10.4" color LED |  |
| 5 | Servo HRV control | High-speed, high-precision servo control |  |
| 6 | Program capacity | 2M |  |
| 7 | Automatic data backup | Backup of machine data |  |
| 8 | Memory Card Edit/Run | DNC online processing |  |
| 9 | Program folder management | Functions of using macro programs and machining programs |  |
| 10 | FSSB high speed rigid tapping | Reduce rigid tapping time and errors |  |
| 11 | backlash compensation | backlash compensation |  |
| 12 | Data transfer via embedded Ethernet | Data transfer via embedded Ethernet |  |
| 13 | USB transfer program to CF card | Data transfer function |  |
| 14 | Spindle load meter | Monitor status such as spindle current |  |
| 15 | AI contour control | Optimum speed and acceleration control for block read-ahead |  |
| 16 | Smooth tolerance control | Determine the appropriate machining speed by commanding allowable tolerances |  |
| 17 | Spindle HRV control | High-speed, high-precision servo control |  |
| 18 | Imperial/Metric conversion | Conversion of machine tool units |  |
| 19 | Dual position feedback | Used when the machine tool is equipped with a fully closed loop grating ruler. |  |
| 20 | FANUC SERVO GUIDE | Servo adjustment for high-speed, high-precision processing in a short time |  |
| 21 | Flexible feed gear | Any DMR |  |
| 22 | Minimum setting unit (0.0001mm, 0.0001deg, 0.00001 inch) | The smallest unit of the system |  |
| 23 | Mirror | Each axis |  |
| 24 | Rigid tapping retraction | Rigid tapping retraction |  |
| 25 | Buffer Register | The system can cache data |  |
| 26 | Positioning | Spindle positioning function for tapping, tool change |  |
| 27 | Straight line interpolation | Linear machining function |  |
| 28 | Arc interpolation | Arc processing function |  |
| 29 | Cylindrical interpolation | Cylindrical processing function |  |
| 30 | Spiral interpolation | Spiral processing function |  |
| 31 | Thread cutting, synchronous feed | Thread cutting, synchronous feed |  |
| 32 | Any angle chamfer/corner R | Any angle chamfer/corner R |  |
| 33 | Fixed cycle for drilling | Fixed cycle for drilling |  |
| 34 | No stop reference point setting | No stop reference point setting |  |
| 35 | Automatic corner magnification | Automatic corner machining override control |  |
| 36 | Scaling | Scaling |  |
| 37 | Plane selection | G17 G18 G19 |  |
| 38 | Programmable mirror | Programmable mirroring of each axis |  |
| 39 | Single program segment | The program can be executed in a single section |  |
| 40 | Small diameter deep hole drilling cycle | Small diameter deep hole drilling cycle |  |
| 41 | Program restarts | After the program is temporarily interrupted, it can be restarted |  |
| 42 | Tool diameter, tool tip radius compensation | Tool parameter compensation |  |
| 43 | Tool life management | Tool life management |  |
| 44 | Backlash compensation | Machine tool axis backlash compensation function |  |
| 45 | location tracking | location tracking |  |
| 46 | Abnormal load detection | Abnormal load detection on each axis |  |
| 47 | Position switch | Program protection function |  |
| 48 | Background editing | Program editing can be performed in edit mode |  |
| 49 | National language display function | System operation interface language display function 10 kinds |  |
| 50 | Self-diagnosis function | Can monitor machine tool status |  |
| 51 | Help function | See system alarm |  |
| 52 | Workpiece coordinate system | G52~G59 |  |
| 53 | Select block to skip | 9 |  |
| 54 | Absolute/incremental instructions | In the same program section can be mixed |  |
| 55 | Fault diagnosis | Can find the current machine fault situation |  |
| 56 | Clear the screen | The screen can be cleared manually or automatically |  |
| 57 | Power consumption monitoring | Can check the machine power consumption |  |

- 4. Equipment recommended oil and grease table

| Use widgets | Name | Capacity | Recommended oil |  | Remark |
| --- | --- | --- | --- | --- | --- |
| hydraulic station | Anti-wear hydraulic oil | 22L | Shell: TELLUS S2 M32 | Winter | Replace it after 3 months of initial use; replace it every 6 months thereafter |
|  |  |  | Great Wall: L-HM 32; Shell: TELLUS S2 M32 | Summer |  |
| Gearbox oil cooler | Anti-wear hydraulic oil | 35L | Shell: TELLUS S2 M32 | Winter | Replace it after 3 months of initial use; replace it every 6 months thereafter |
|  |  |  | Great Wall: L-HM 32; Shell: TELLUS S2 M32 | Summer |  |
| Spindle water cooling machine | Antifreeze coolant | 15L | BASF (BASF): Gulishun G48; Mobil: -45℃ antifreeze coolant |  | Replace every 3 months |
| Lubrication pump | Extreme pressure lithium grease | 2L | Great Wall: 00#; Shell: Gadus S2 V220 00# | Winter | Refill when the oil level is below the minimum level line |
|  |  |  | Great Wall: 00#; Shell: Gadus S2 V220 00# | Summer |  |

5、Others

- Matters not covered in this agreement shall be resolved through negotiation between the two parties.

- This agreement is an attachment to the contract and will take effect simultaneously with the contract after being signed by Party A and Party B.

- This agreement is made in duplicate, with Party A and Party B each holding one copy