T-1585L vertical machining center

Technology Agreement

| Party A（buyer）： | Represent： date： |
| --- | --- |
| Party B（seller）： Shenzhen Genesis Machinery Co., Ltd. | Represent： date： |

Directory

1	Equipment technical description, parameters, and configuration	1

1.1.	Equipment technical description	1

	The whole machine structure	1

	The foundation parts	2

	The spindle	2

	The screw	2

	Guide rails	2

	The lubrication system	2

	Chip removal	2

	CNC system	3

	Working conditions	3

1.2.	Equipment Parameter and configuration	4

1.2.1.	Basic Parameters	4

1.2.2.	Standard configuration	6

1.2.3.	Customer optional configuration	7

2	Core component brands	8

3	System main function table	9

4	Recommended oil and grease table	14

5	Quality assurance	17

6	Others	18

# Equipment technical description, parameters, and configuration

## Equipment technical description

T-1585L vertical machining center is a medium-sized machine tool. With a workpiece each clamping cycle, it is capable of performing automatically and continuously various machining operations such as milling, drilling, boring, expanding, reaming, countersinking, tapping, and other processes. The machine tool is suitable for processing of various types of small and medium-sized batches of complex parts such as medium-sized boxes, plates, disks, valves, and shells.

- The whole machine structure: The structure of the whole machine is fixed column type and cross slide structure. The Y-axis of the machine tool adopts a four-guide guide rail design, which ensures heavy-load capacity during processing and ensure processing accuracy. The whole machine is fully enclosed for protection, as well as the machine tool has beautiful appearance and is easy to operate and maintain.

- The foundation parts: The basic large parts are made of high-quality resin sand molding and high-strength cast iron materials to ensure high rigidity and stable accuracy of the machine tool.

- The spindle: The spindle adopts a high-speed spindle unit from Taikan, coming standard with a belt-drive BT50-6000 r/min spindle with high cutting efficiency, and reduces thermal deformation of spindle.

- The front end of spindle adopt labyrinth design with an air curtain protection function to prevent cutting fluid from entering the spindle bearing.

- The spindle has a central air blowing function, equipped with an automatic constant-temperature circulating water cooling device, which reduces the temperature rise and thermal deformation of the spindle and maintains high machining accuracy.

- The screw: The three-axis feeding adopts a direct connection drive between the motor and the high-precision ball screw, and the large-diameter C3 grade ball screw is mounted by pre-stretching to further improve the transmission rigidity and positioning accuracy.

- Guide rails: X and Y-axis guide rails adopt 45# linear guide rails to load, featuring high rigidity, minimal noise, and reduced friction. Z-axis guide rails adopt cast iron guide rails that are ground after quenching and hardening treatment and veneered with PTFE plastic, featuring good vibration absorption, large contact surfaces, high load carrying capacity and heavy cutting.

- The lubrication system: The guide rails and screws of the machine tool adopt a timed and quantitative lubrication system, which can adjust the oil supply cycle and amount of each lubrication point, and realize fully automatic centralized lubrication through electrical system control.

- Chip removal: The chip removal of the machine tool adopts a built-in double screw conveyor and a front chain plate type chip conveyor. It has a high degree of automation and reduces manual cleaning time. It is automated, user-friendly and highly efficient.

- The water tank of the machine adopts the structure of multi-layer filtration system to provide good cutting fluid for workpiece machining.

- CNC system: The machine is equipped with the high-performance FANUC 0i-MF PULS (5) CNC system, which guarantees the stability of the machine control, as well as the CNC machining functions and auxiliary functions requested by the user.

### Working conditions

- Three-phase AC power supply: 380V±5%; 50Hz±1Hz;

- Ambient temperature: need to keep the ambient temperature 17 ~ 25 ℃, If the precision requirements of processing parts are not high, can be relaxed to 5 ~ 40 ℃. Storage or transport temperature -20～60℃

- Room temperature change: The ambient temperature changes within ±2°C within 24 hours.

- Relative humidity: Within 75% (no condensation);

- Altitude: Below 1000m;

- Vibration: Below 0.2G；

- Foundation requirements: bearing capacity above 50KN/m²；

- Keep away from light sources, vibration sources, and heat sources, and away from high-frequency generators, dischargers, welding machines, etc. to avoid electrical interference that may cause machine tool NC system malfunctions.

- If the voltage in the area of use is not stable, the machine tool should be equipped with a regulated power supply.

- The machine tool should have reliable grounding: the grounding wire is a copper wire, the wire diameter should not be less than 16 mm ², and the grounding resistance should be less than 4Ω.

- In order to ensure the normal working performance of equipment, if the air compressed by the air source does not meet the cleanliness requirements, an air source purification device should be equipped before the machine tool air intake.

## Equipment Parameter and configuration

### Basic Parameters

| Name |  |  | Unite | Parameter | Remark |
| --- | --- | --- | --- | --- | --- |
| Stroke range | Worktable stroke(X-axis) |  | mm | 1550 |  |
|  | Saddle stroke(Y-axis) |  | mm | 850 |  |
|  | Spindle box stroke(Z-axis) |  | mm | 850 |  |
|  | The distance from the spindle end face to the worktable |  | mm | 120～970 |  |
|  | Distance from spindle center to column guide |  | mm | 935 |  |
| Worktable | Worktable size |  | mm | 1700*850 |  |
|  | Worktable load |  | Kg | 1500 |  |
|  | T-shaped slot (number of slots-slot width*spacing) |  | mm | 5-18*125+2-18*100 |  |
| Spindle | Motor power (rated/short time) |  | kw | 15/18.5 |  |
|  | Motor torque (rated/short time) |  | N.m | 143/191 |  |
|  | rotation speed range |  | r/min | 50～6000(Belt） |  |
|  | Spindle diameter |  | mm | φ190 |  |
|  | Handle specifications |  | / | BT50 |  |
|  | blind rivet specification |  | / | P50T-I（MAS403） |  |
| Guide rail | X-axis |  | mm | 2-45 roller |  |
|  | Y-axis |  | mm | 4-45 roller |  |
|  | Z-axis |  | / | Rectangular guide rail |  |
| Drive | Screw X/Y/Z |  | mm | 50*12/50*12/50*12 |  |
|  | Motor power X/Y/Z |  | kw | 3.0/3.0/3.0 |  |
| Tool magazine | Tool magazine capacity |  | T | 24 |  |
|  | tool weight |  | kg | 15 |  |
|  | Tool length |  | mm | 300 |  |
|  | Maximum diameter (full tool/adjacent empty tool) |  | mm | Φ105/Φ210 |  |
| Speed | Cutting feed speed range |  | mm/min | 1-10000 |  |
|  | Fast-moving speed of X/Y/Z-axis |  | m/min | 24/24/15 |  |
| Machine tool accuracy | Positioning accuracy (X/Y/Z) |  | mm | 0.015 | GB/T18400.4 （total Stroke） |
|  | Repeated positioning accuracy（X/Y/Z） |  | mm | 0.008 |  |
| Other | Z-axis counterweight |  | / | Have |  |
|  | CNC system |  |  | FANUC 0i-MF Plus(5) |  |
|  | Gas source | flow | L/min | ≥280（ANR) |  |
|  |  | air pressure | MPa | 0.6~0.8 |  |
|  | Equipment capacity |  | KVA | 40 |  |
|  | Cooling box volume |  | L | 400 |  |
|  | Machine tool appearance dimensions (length*width*height) |  | mm | 4190*3380*3652 | Without chain row size |
|  | Main unit weight (approx.) |  | Kg | 12400 |  |

#: The appearance dimensions of the machine tool may change due to configuration without further notice.

### Standard configuration

| No | Name | Amount | Remark |
| --- | --- | --- | --- |
| 1 | FANUC 0i-MF Plus(5) CNC system | 1 |  |
| 2 | Operation panel | 1 |  |
| 3 | Pneumatic system | 1 |  |
| 4 | Air gun | 1 |  |
| 5 | Automatic lubrication system | 1 |  |
| 6 | Spindle cooling system | 1 | Water cooling |
| 7 | Workpiece cooling system | 1 |  |
| 8 | Built-in spiral chip conveyor | 2 |  |
| 9 | Front chain plate chip conveyor + chip catcher trolley | 1 |  |
| 10 | Full protection | 1 |  |
| 11 | Three-color light | 1 |  |
| 12 | Lighting devices | 1 |  |
| 13 | Basic installation kit | 1 |  |
| 14 | Randomized sets of standard technical documents | 1 |  |
| 15 | BT50-24T arm-type tool magazine | 1 |  |

### Customer optional configuration

| No | Name and specification | Selection | Remark |
| --- | --- | --- | --- |
| 1 |  | √ |  |
| 2 |  |  |  |
| 3 |  |  |  |

Note: If you select this configuration, mark √ in the corresponding item box

# Core component brands

| Core component brands and specification |  |  |  |
| --- | --- | --- | --- |
| No | Component name | Brands | Precision grade |
| 1 | Controller systems | FANUC 0i-MF Plus(5) system | High-performance grade |
| 2 | Spindle unit | Taikan | High-Precision grade |
| 3 | Three-axis bearings | Japan NSK/GermanyFAG | P4 grade |
| 4 | Three-axis screw | Taiwan PMI/Hiwin | C3 grade |
| 5 | X,Y-axis roller linear guide rails | Taiwan PMI/Hiwin | P grade |
| 6 | Main electrical components | French Schneider |  |
| 7 | 24T arm-type tool magazine | Taikan |  |
| 8 | Main pneumatic components | Japan SMC |  |
| 9 | Server Driver | Japan FANUC |  |
| Note: When supply is insufficient, other brands of products will be used instead, but the quality and grade will not be lower than the listed configurations. |  |  |  |

# System main function table

（Some functions need to be combined with the configuration of the host to be realized）

| No | Function | clarification | Remark |
| --- | --- | --- | --- |
| 1 | Number of control paths | 1 |  |
| 2 | Maximum total number of control axes (feed axis + spindle) | 6 |  |
|  | Maximum number of feed axes | 5 |  |
|  | Maximum number of spindles | 1 |  |
| 3 | Control the number of axes simultaneously | 4 |  |
| 4 | Display unit | 10.4＂ |  |
| 5 | Servo HRV control | High-speed, high-precision servo control |  |
| 6 | Program capacity | 2M |  |
| 7 | I/O Link | 1024/1024 |  |
| 8 | Automatic data backup | Backup of machine data |  |
| 9 | Memory Card Edit/Run | DNC online processing |  |
| 10 | Program folder management | Functions for using macro programs and machining programs |  |
| 11 | FSSB High-speed rigid tapping | Reduced rigid tapping time and errors |  |
| 12 | Intelligent backlash compensation | Backlash compensation |  |
| 13 | PMC axis control |  |  |
| 14 | Data transfer via embedded Ethernet | Data transfer function via Ethernet |  |
| 15 | USB transfer program to CF card | Data transfer function |  |
| 16 | Spindle Intelligent load meter | Monitoring spindle current and other status |  |
| 17 | Spindle HRV control | High-speed, high-precision servo control |  |
| 18 | AI contour controlⅠ | Pre-reading of program segments within 40 segments (others are not optional) |  |
| 19 | Bell-shaped acceleration and deceleration before read-ahead interpolation |  |  |
| 20 | Intelligent overlap | Achieving reduction of cycle times for machining parts |  |
| 21 | Pre-processing preparation support function |  |  |
| 22 | Imperial/Metric Conversion | Conversion of machine tool units |  |
| 23 | HRV3 control | High-speed, high-precision servo control |  |
| 24 | Flexible feed gear | Arbitrary DMR |  |
| 25 | Minimum setting unit（0.0001mm、0.0001deg、0.00001 inch） | The smallest unit of the system |  |
| 26 | Mirror | Each axis |  |
| 27 | Rigid tapping retraction | Rigid tapping retraction |  |
| 28 | Buffer register | The system can cache data |  |
| 29 | Positioning |  |  |
| 30 | Linear interpolation | Linear machining function |  |
| 31 | Arc interpolation | Arc processing function |  |
| 32 | Spiral interpolation | Spiral processing function |  |
| 33 | Flip of the rotary axis | Flip of the rotary axis |  |
| 34 | Arbitrary Angle Chamfer/Corner R | Arbitrary Angle Chamfer/Corner R |  |
| 35 | Fixed cycle for drilling | Fixed cycle for drilling |  |
| 36 | No block reference point setting | No block reference point setting |  |
| 37 | Automatic cornering magnification | Automatic corner processing ratio control |  |
| 38 | Scaling | Scaling |  |
| 39 | Plane selection | G17 G18 G19 |  |
| 40 | Programmable mirror | Programmable mirroring of each axis |  |
| 41 | Single program segment | Single-segment of the program can be executed |  |
| 42 | Program restarts | After the program is temporarily interrupted, it can be restarted |  |
| 43 | Tool diameter, tool nose radius compensation | Tool parameter compensation |  |
| 44 | Tool life management | Tool life management |  |
| 45 | Tool life management extension | Tool life management extension |  |
| 46 | Spindle stops accurately |  |  |
| 47 | Position tracking | Position tracking |  |
| 48 | Abnormal load detection | Abnormal load detection for each axis |  |
| 49 | Position switch | Program protection function |  |
| 50 | Backstage Editing | Program editing can be performed in edit mode |  |
| 51 | National language display function | System operation interface language display function 10 kinds |  |
| 52 | Self-diagnosis function | Can monitor machine tool status |  |
| 53 | Help function | Please refer to System Alarm |  |
| 54 | Nano interpolation | Nano interpolation |  |
| 55 | Workpiece coordinate system | G52~G59 |  |
| 56 | Absolute/Incremental Instructions | Can be mixed in the same program section |  |
| 57 | Fault diagnosis | Can find current machine tool fault conditions |  |
| 58 | Clear screen | Manual or automatic clearing of the screen |  |
| 59 | Power consumption monitoring | Can check the power consumption of machine tools |  |

# Recommended oil and grease table

（Refer to this table according to the actual configuration of the equipment）

| Lubrication part |  | Amount | Recommended oil |  | Remark |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Room temperature 10～40℃ | Room temperature 5～10℃ |  |
| Guide screw | Oil lubrication | 2-4L | Guide rail oil 68# | Guide rail oil 32# | 1. Check before work every day. When warning signs appear, please refuel. 2. Pay attention to changing the oil when the temperature is low. 3. When using canned grease, the brand of canned grease needs to be consistent with the manufacturer of the oil injector. |
|  | Grease lubrication | 0.7-0.8L | Lithium grease 000# |  |  |
| Spindle screw cooler | Water cooler | 15-35L | BASF G48 Antifreeze（No media is allowed to be added） |  | 1. Check before work every day. When the warning signal appears, please add antifreeze or refuel. 2. Pay attention to changing the oil when the temperature is low. |
|  | Oil cooler | 20-40L | 10# Great Wall Spindle oil | 5# Great Wall Spindle oil |  |
| Hydraulic station |  | 25L | Hydraulic oil HV32# | Hydraulic oil HV20# | 1. Inspect daily before work and replace the oil with new oil every year. 2. Pay attention to changing the oil when the temperature is low. |
| Booster cylinder |  | 0.1-0.2L | Hydraulic oil 32# | Hydraulic oil 20# | 1. Check before work every day and refill the oil cup when it is only 1/3 full. 2. Pay attention to changing the oil when the temperature is low. |
| Cutting fluid |  | 100-350L | According to the workpiece material and processing conditions, select the appropriate cutting fluid (based on the actual oil tank capacity on site)) |  | When the room temperature is lower than 5℃, please check whether there is ice in the fuel tank before starting the machine. |
| Fourth axis |  | 0.8~1L | Gear Oil 150# | Gear Oil 100# | 1. Check daily before work and replace with new oil every six months. 2. Pay attention to changing the oil when the temperature is low. |
| Tool changing arm system |  | 7L | Gear Oil 100# | 15W-40 Antifreeze diesel engine oil | 1. Check daily before work, and replace with new oil every 2400 working hours or every six months. 2. Pay attention to changing the oil when the temperature is low. |

# Quality assurance

- The quality guarantee period is 12 months from the date of final acceptance.

- During the warranty period, if the buyer uses the equipment or machine tool under normal conditions in accordance with the instruction manual, the seller will be responsible for any damage caused by defects in factory design or manufacturing. During the above warranty period, if there is a problem with the equipment, the seller will make correction suggestions to the buyer; if the buyer still cannot solve the problem according to the seller's instructions and system description, the seller shall dispatch a maintenance engineer free of charge within 2 working days after receiving the buyer's notice. Arrive at the buyer's factory.

- During the warranty period, the seller shall provide the buyer with timely maintenance services and necessary spare parts free of charge. It is verified that the failure loss caused by the buyer's improper operation, unreasonable cutting parameter selection or self-disassembly and modification, and other non-manufacturing quality reasons, as well as the damage to the appearance or interior of the machine tool caused by excessive storage time, are not covered by the above warranty, and the buyer must pay for relevant maintenance cost.

- After the warranty period, the seller will maintain the machine tool for life and provide free online services and technical guidance. Provide timely supply of spare parts to buyers at favorable prices. If it is necessary to send someone to the site for repairs, the corresponding fees will be charged according to the seller's standards.

# Others

1. Matters not covered in this agreement shall be resolved through negotiation between the two parties.

2. This agreement is an attachment to the contract and will take effect simultaneously with the contract after it is signed by Party A and Party B.

3. This agreement is made in duplicate, with Party A and Party B each holding one copy.