T-V1285W vertical machining center

Technology Agreement

| Party A（buyer）： | Represent： date： |
| --- | --- |
| Party B（seller）： Shenzhen Genesis Machinery Co., Ltd. | Represent： date： |

（The pictures are for reference only）

Directory

1	Equipment technical description, parameters, and configuration	1

1.1.	Equipment technical description	1

	The whole machine structure	1

	The foundation parts	1

	Guide rails	1

	The screw	1

	The tool magazine	1

	The spindle	2

	The balance device:	2

	The lubrication system	2

	Chip removal	2

	CNC system	2

	Working conditions	2

Equipment Parameter and configuration	3

1.1.1.	Basic Parameters	3

1.1.2.	Standard configuration	5

1.1.3.	Customer optional configuration	6

2	Core component brands	6

3	System main function table	7

4	Recommended oil and grease table	11

5	Quality assurance	14

6	Others	15

# Equipment technical description, parameters, and configuration

## Equipment technical description

T-V1285W linear guide rail type vertical machining center is a medium-sized machine tool. With a workpiece each clamping cycle, it is capable of performing automatically and continuously various machining operations such as milling, drilling, boring, expanding, reaming, countersinking, tapping, and other processes. The machine tool is suitable for processing of various types of small and medium-sized batches of complex parts such as medium-sized boxes, plates, disks, valves, and shells. The machine tool adopts high-precision linear guide rails and screws with the better dynamic response, which achieves high-speed cutting and no crawling at low speed. It is widely used in precision parts, 5G products, hardware, auto parts, and medical equipment industries.

- The whole machine structure: The structure of the whole machine is fixed column type and cross slide structure. The body of the machine adopts a large and stable base, as well as High-rigidity long-span herringbone column, featuring no vibration or deformation during high-speed cutting.

- The whole machine is fully enclosed for protection, the machine tool has beautiful appearance and is easy to operate and maintain.

- The foundation parts: The basic large parts are made of high-quality resin sand molding and high-strength cast iron materials to ensure high rigidity and stable accuracy of the machine tool.

- Guide rails: X and Y-axis guide rails adopt 45# roller linear guide rails, Z-axis guide rails adopt 55# roller guide to load, featuring high rigidity, minimal noise, and reduced friction.

- The screw: large-diameter C3 grade screw with pre-tensioned structure eliminates some thermal extension. The screw bearing is self-lubricating to extend the lifespan of screw bearing

- The tool magazine: The tool magazine adopts 24T arm-type tool magazine from Taikan, which has reliable quality and stable performance.

- The spindle: The spindle adopts a high-speed spindle unit from Taikan, equipped with a BT40-12000 r/min direct-connected head. The front end of spindle adopt labyrinth design with an air curtain protection function to prevent cutting fluid from entering the spindle bearing.

- The balance device: The spindle box movement (Z-axis) is equipped with a Nitrogen balance device for better stability at high speeds. The precise ratio of counterweight to spindle box weight results in optimal machining characteristics. And the Z-axis drive motor has good load characteristics.

- The lubrication system: Equipped with positive displacement forced lubricating oil system to ensure good lubrication of all moving parts of the machine.

- Chip removal: The chip removal of the machine tool comes standard with both sides of the slope rear chip removal, and the rear water tank design is user-friendly and highly efficient. The double-layer water tank filtration system structure provides good cutting fluid for workpiece processing.

- CNC system: The machine is equipped with the high-performance FANUC 0i-MF PULS (5) CNC system, which guarantees the stability of the machine control, as well as the CNC machining functions and auxiliary functions requested by the user.

### Working conditions

- Three-phase AC power supply: 380V±5%; 50Hz±1Hz; main power cord, ground wire 10mm² or larger.

- Ambient temperature: 5 ~ 40 ℃

- Relative humidity: ≤80%

- Keep away from light sources, vibration sources, and heat sources, and away from high-frequency generators, dischargers, welding machines, etc. to avoid electrical interference that may cause machine tool NC system malfunctions.

## Equipment Parameter and configuration

### Basic Parameters

| Name |  |  |  | Unite | Parameter | Remark |
| --- | --- | --- | --- | --- | --- | --- |
| Stroke range | Worktable stroke(X-axis) |  |  | mm | 1200 |  |
|  | Saddle stroke(Y-axis) |  |  | mm | 850 |  |
|  | Spindle box stroke(Z-axis) |  |  | mm | 700 |  |
|  | The distance from the spindle end face to the worktable |  |  | mm | 80～780 |  |
|  | Distance from spindle center to column guide |  |  | mm | 921 |  |
| Worktable | Worktable size |  |  | mm | 1300*850 |  |
|  | Worktable load |  |  | Kg | 1000 |  |
|  | T-shaped slot (number of slots-slot width*spacing) |  |  | mm | 7-18*125 |  |
| Spindle | Motor power (rated/short time) |  |  | kw | 11/15 |  |
|  | Motor torque (rated/short time) |  |  | N.m | 52.8/118 |  |
|  | rotation speed range |  |  | r/min | 50～12000 (direct-connected） |  |
|  | Spindle diameter |  |  | mm | φ150 |  |
|  | Handle specifications |  |  | / | BT40 |  |
|  | blind rivet specification |  |  | / | P40T-I（MAS403） |  |
| Guide rail | X-axis |  |  | mm | 2-45 roller |  |
|  | Y-axis |  |  | mm | 2-45 roller |  |
|  | Z-axis |  |  | / | 2-55 roller |  |
| Tool magazine | Tool magazine capacity |  |  | T | 24 |  |
|  | tool weight |  |  | kg | 7 |  |
|  | Tool length |  |  | mm | 250 |  |
|  | Maximum diameter (full tool/adjacent empty tool) |  |  | mm | Φ75/Φ150 |  |
| Drive | Screw X/Y/Z |  |  | mm | 4012/4012/5010 |  |
|  | Motor power X/Y/Z |  |  | kw | 3.0/3.0/3.0 |  |
| Speed | Cutting feed speed range |  |  | mm/min | 1-10000 |  |
|  | Fast-moving speed of X/Y/Z-axis |  |  | m/min | 24/24/16 |  |
| Machine tool accuracy | Positioning accuracy (X/Y/Z) |  |  | mm | 0.01 | GB/T18400.4 （total Stroke） |
|  | Repeated positioning accuracy（X/Y/Z） |  |  | mm | 0.007 |  |
| Other | CNC system |  |  |  | FANUC 0i-MF Plus(5) |  |
|  | Gas source | flow |  | L/min | ≥280（ANR) |  |
|  |  | air pressure |  | MPa | 0.6~0.8 |  |
|  | Equipment capacity |  |  | KVA | 40 |  |
|  | Cooling box volume |  |  | L | 350 |  |
|  | Machine tool appearance dimensions (length*width*height) |  |  | mm | 3000×4080×3200 | Corresponds to X*Y*Z axis |
|  | Main unit weight (approx.) |  |  | Kg | 9200 |  |

#: The appearance dimensions of the machine tool may change due to configuration without further notice.

### Standard configuration

| No | Name | Amount | Remark |
| --- | --- | --- | --- |
| 1 | FANUC 0i-MF Plus(5) CNC system | 1 |  |
| 2 | Operation panel | 1 |  |
| 3 | Pneumatic system | 1 |  |
| 4 | Air gun | 1 |  |
| 5 | Automatic lubrication system | 1 |  |
| 6 | Workpiece cooling system | 1 |  |
| 7 | Rear water tank | 1 |  |
| 8 | Three-axis built-in encoder feedback system | 3 |  |
| 9 | Three-axis protective cover | 3 |  |
| 10 | Full protection | 1 |  |
| 11 | Three-color light | 1 |  |
| 12 | Lighting devices | 1 |  |
| 13 | Basic installation kit | 1 |  |
| 14 | Randomized sets of standard technical documents | 1 |  |
| 15 | Spindle water cooling system | 1 |  |

### Customer optional configuration

| No | Name and specification | Selection | Remark |
| --- | --- | --- | --- |
| 1 |  |  |  |
| 2 |  |  |  |
| 3 |  |  |  |

Note: If you select this configuration, mark √ in the corresponding item box

# Core component brands

| Core component brands and specification |  |  |  |
| --- | --- | --- | --- |
| No | Component name | Brands | Precision grade |
| 1 | Controller systems | FANUC 0i-MF Plus(5) system | High-performance grade |
| 2 | Spindle unit | Taikan | High-Precision grade |
| 3 | Three-axis bearings | Japan NSK/GermanyFAG | P4 grade |
| 4 | Three-axis screw | Taiwan PMI/Hiwin | C3 grade |
| 5 | roller linear guide rails | Taiwan PMI/Hiwin | P grade |
| 6 | Main electrical components | French Schneider |  |
| 7 | 24T arm-type tool magazine | Taikan |  |
| 8 | Main pneumatic components | Japan SMC |  |
| 9 | Servo Driver | Japan FANUC |  |
| Note: When supply is insufficient, other brands of products will be used instead, but the quality and grade will not be lower than the listed configurations. |  |  |  |

# System main function table

（Some functions need to be combined with the configuration of the host to be realized）

| No | Function | clarification |
| --- | --- | --- |
| 1 | Number of control paths | 1 |
| 2 | Maximum total number of control axes (servo axis + spindle) | 6 |
|  | Maximum number of servo axes | 5 |
|  | Maximum number of spindles | 1 |
| 3 | Control the number of axes simultaneously | 4 |
| 4 | Display unit | 10.4" color LED |
| 5 | Servo HRV control | High-speed, high-precision servo control |
| 6 | Program capacity | 512K（1280m） |
| 7 | I/O Link i | 1024/1024 |
| 8 | Automatic data backup | Backup of machine data |
| 9 | Memory Card Edit/Run | DNC online processing |
| 10 | Program folder management | Functions for using macro programs and machining programs |
| 11 | FSSB High-speed rigid tapping | Reduced rigid tapping time and errors |
| 12 | Intelligent backlash compensation | Backlash compensation |
| 13 | PMC axis control | Used when the tool magazine is a servo magazine |
| 14 | Data transfer via embedded Ethernet | Data transfer function via Ethernet |
| 15 | USB transfer program to CF card | Data transfer function |
| 16 | Servo Guide Mate | Servo optimization |
| 17 | Spindle Intelligent load meter | Monitoring spindle current and other status |
| 18 | AI contour control | Optimal speed and acceleration control with program segment pre-reading |
| 19 | Smooth tolerance control | Determine the appropriate machining speed through the tolerances allowed by the directive |
| 20 | Intelligent overlap | Achieving reduction of cycle times for machining parts |
| 21 | Spindle HRV control | High-speed, high-precision servo control |
| 22 | Imperial/Metric Conversion | Conversion of machine tool units |
| 23 | HRV2 control | High-speed, high-precision servo control |
| 24 | HRV3 control | High-speed, high-precision servo control |
| 25 | FANUC SERVO GUIDE | Servo adjustment for high-speed, high-precision processing in a short time |
| 26 | Flexible feed gear | Arbitrary DMR |
| 27 | Minimum setting unit（0.0001mm、0.0001deg、0.00001 inch） | The smallest unit of the system |
| 28 | Mirror | Each axis |
| 29 | Rigid tapping retraction | Rigid tapping retraction |
| 30 | Buffer register | The system can cache data |
| 31 | Positioning | Spindle positioning function for tapping and tool changing |
| 32 | Linear interpolation | Linear machining function |
| 33 | Circular arc interpolation | Circular arc processing function |
| 34 | Spiral interpolation | Spiral processing function |
| 35 | Thread cutting, synchronous feed | Thread cutting, synchronous feed |
| 36 | Flip of the rotary axis | Flip of the rotary axis |
| 37 | Arbitrary Angle Chamfer/Corner R | Arbitrary Angle Chamfer/Corner R |
| 38 | Fixed cycle for drilling | Fixed cycle for drilling |
| 39 | No block reference point setting | No block reference point setting |
| 40 | Automatic cornering magnification | Automatic corner processing ratio control |
| 41 | Scaling | Scaling |
| 42 | Plane selection | G17 G18 G19 |
| 43 | Programmable mirror | Programmable mirroring of each axis |
| 44 | Single program segment | Single-segment of the program can be executed |
| 45 | Program restarts | After the program is temporarily interrupted, it can be restarted |
| 46 | Tool diameter, tool nose radius compensation | Tool parameter compensation |
| 47 | Tool life management | Tool life management |
| 48 | Tool life management extension | Tool life management extension |
| 49 | backlash compensation | Machine tool axis backlash compensation function |
| 50 | Position tracking | Position tracking |
| 51 | Abnormal load detection | Abnormal load detection for each axis |
| 52 | Position switch | Program protection function |
| 53 | Backstage Editing | Program editing can be performed in edit mode |
| 54 | National language display function | System operation interface language display function 10 kinds |
| 55 | Self-diagnosis function | Can monitor machine tool status |
| 56 | Help function | Please refer to System Alarm |
| 57 | Nano interpolation | Nano interpolation |
| 58 | Workpiece coordinate system | G52~G59 |
| 59 | Absolute/Incremental Instructions | Can be mixed in the same program section |
| 60 | Fault diagnosis | Can find current machine tool fault conditions |
| 61 | Clear screen | Manual or automatic clearing of the screen |
| 62 | Power consumption monitoring | Can check the power consumption of machine tools |

# Recommended oil and grease table

（Refer to this table according to the actual configuration of the equipment）

| Lubrication part |  | Amount | Recommended oil |  | Remark |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Room temperature 10～40℃ | Room temperature 5～10℃ |  |
| Guide screw | Oil lubrication | 2-4L | Guide rail oil 68# | Guide rail oil 32# | 1. Check before work every day. When warning signs appear, please refuel. 2. Pay attention to changing the oil when the temperature is low. 3. When using canned grease, the brand of canned grease needs to be consistent with the manufacturer of the oil injector. |
|  | Grease lubrication | 0.7-0.8L | Lithium grease 000# |  |  |
| Spindle screw cooler | Water cooler | 15-35L | BASF G48 Antifreeze（No media is allowed to be added） |  | 1. Check before work every day. When the warning signal appears, please add antifreeze or refuel. 2. Pay attention to changing the oil when the temperature is low. |
|  | Oil cooler | 20-40L | 10# Great Wall Spindle oil | 5# Great Wall Spindle oil |  |
| Hydraulic station |  | 25L | Hydraulic oil HV32# | Hydraulic oil HV20# | 1. Inspect daily before work and replace the oil with new oil every year. 2. Pay attention to changing the oil when the temperature is low. |
| Booster cylinder |  | 0.1-0.2L | Hydraulic oil 32# | Hydraulic oil 20# | 1. Check before work every day and refill the oil cup when it is only 1/3 full. 2. Pay attention to changing the oil when the temperature is low. |
| Cutting fluid |  | 100-350L | According to the workpiece material and processing conditions, select the appropriate cutting fluid (based on the actual oil tank capacity on site)) |  | When the room temperature is lower than 5℃, please check whether there is ice in the fuel tank before starting the machine. |
| Fourth axis |  | 0.8~1L | Gear Oil 150# | Gear Oil 100# | 1. Check daily before work and replace with new oil every six months. 2. Pay attention to changing the oil when the temperature is low. |
| Tool changing arm system |  | 7L | Gear Oil 100# | 15W-40 Antifreeze diesel engine oil | 1. Check daily before work, and replace with new oil every 2400 working hours or every six months. 2. Pay attention to changing the oil when the temperature is low. |

# Quality assurance

- The quality guarantee period is 12 months from the date of final acceptance.

- During the warranty period, if the buyer uses the equipment or machine tool under normal conditions in accordance with the instruction manual, the seller will be responsible for any damage caused by defects in factory design or manufacturing. During the above warranty period, if there is a problem with the equipment, the seller will make correction suggestions to the buyer; if the buyer still cannot solve the problem according to the seller's instructions and system description, the seller shall dispatch a maintenance engineer free of charge within 2 working days after receiving the buyer's notice. Arrive at the buyer's factory.

- During the warranty period, the seller shall provide the buyer with timely maintenance services and necessary spare parts free of charge. It is verified that the failure loss caused by the buyer's improper operation, unreasonable cutting parameter selection or self-disassembly and modification, and other non-manufacturing quality reasons, as well as the damage to the appearance or interior of the machine tool caused by excessive storage time, are not covered by the above warranty, and the buyer must pay for relevant maintenance cost.

- After the warranty period, the seller will maintain the machine tool for life and provide free online services and technical guidance. Provide timely supply of spare parts to buyers at favorable prices. If it is necessary to send someone to the site for repairs, the corresponding fees will be charged according to the seller's standards.

# Others

1. Matters not covered in this agreement shall be resolved through negotiation between the two parties.

2. This agreement is an attachment to the contract and will take effect simultaneously with the contract after it is signed by Party A and Party B.

3. This agreement is made in duplicate, with Party A and Party B each holding one copy.