T-V16 vertical machining center

Technology Agreement

| Party A（buyer）： | Represent： date： |  |
| --- | --- | --- |
| Party B（seller）： Shenzhen Genesis Machinery Co., Ltd. | Represent： date： |  |

（The pictures are for reference only）

Directory

1.	Equipment technical description, parameters, and configuration	1

1.1.	Equipment technical description	1

	The whole structure	1

	The spindle	1

	The lubrication system	1

	Chip removal	1

	Rapid movement speed	2

	Working conditions	2

1.2.	Equipment Parameter and configuration	2

1.2.1.	Basic Parameters	2

1.2.2.	Standard configuration	5

1.2.3.	Customer optional configuration	6

2.	Core component brands	7

3.	System main function table	7

4.	Recommended oil and grease table	13

5.	Routine maintenance and upkeep of machine tools	14

6.	Quality assurance	14

7.	Others	15

# Equipment technical description, parameters, and configuration

## Equipment technical description

T-V16 vertical machining center, X, Y, and Z three-axis screws all adopt precise ball screws, and the three-axis guide rails adopt precise linear guide rails; the three axes adopt absolute value type servo motors. The spindle adopts a precision-grade spindle, with a direct-coupled transmission structure, and the standard maximum speed reaches 12,000 rpm; the cast iron adopts the Meehanite casting process and is completely heat treated to eliminate internal stress. The machine tool has high speed and high rigidity, integrating drilling, tapping, milling and other processing. It is mainly used in medium and large boxes, frames, plate parts, disc parts, and shells in general industries such as 5G. Light processing.

- The whole structure: The structure of the whole machine is fixed column type and cross slide structure. The body of the machine adopts a large and stable base, as well as High-rigidity long-span herringbone column, featuring no vibration or deformation during high-speed cutting.

- The spindle: The spindle motor and spindle is directly driven to reduce errors in high-speed tapping, and the high output force characteristics can be greatly utilized in high-speed and efficient processing;

- The high-speed spindle has fast response speed and uses ceramic ball bearings, which greatly extends the service life of the spindle;

- The lubrication system: Equipped with positive displacement forced lubrication system to ensure good lubrication of all moving parts of the machine.

- Chip removal: The large inclined chip removal angle of the machine makes it possible to remove chips quickly and accelerates the return speed of cutting fluid.

- Equipped with high-speed rigid tapping.

- By loading a high-speed PLC engine, the response speed is improved, the cycle is shortened, the processing detail parameters are optimized, the smoothness of the feed is improve

- Rapid movement speed: The rapid movement speed of the machine tool X30/Y36/Z36m/min shortens the processing time and improves the processing efficiency

### Working conditions

- Three-phase AC power supply: 380V±5%; 50Hz±1Hz; main power cord, ground wire 10mm² or larger.

- Ambient temperature: need to keep the ambient temperature 17 ~ 25 ℃, If the precision requirements of processing parts are not high, can be relaxed to 5 ~ 40 ℃. Storage or transport temperature -20～60℃

- Room temperature change: The ambient temperature changes within ±2°C within 24 hours.

- Relative humidity: Within 75% (no condensation);

- Altitude: Below 1000m;

- Vibration: Below 0.2G；

- Foundation requirements: bearing capacity above 50KN/m²；

- Keep away from light sources, vibration sources, and heat sources, and away from high-frequency generators, dischargers, welding machines, etc. to avoid electrical interference that may cause machine tool NC system malfunctions.

## Equipment Parameter and configuration

### Basic Parameters

| Name |  |  | Unite | Parameter | Remark |
| --- | --- | --- | --- | --- | --- |
| Stroke range | Worktable stroke(X-axis) |  | mm | 1600 |  |
|  | Saddle stroke(Y-axis) |  | mm | 650 |  |
|  | Spindle box stroke(Z-axis) |  | mm | 590 |  |
|  | The distance from the spindle end face to the worktable |  | mm | 100～690 |  |
|  | Distance from spindle center to column guide surface |  | mm | 707 |  |
| Worktable | Worktable size |  | mm | 1700*650 |  |
|  | Worktable load |  | Kg | 800 |  |
|  | T-shaped slot (number of slots-slot width*spacing) |  | mm | 5-18*125 |  |
| Spindle | Motor power (rated/short time S3/S6 25%) |  | kw | 7.5/11 |  |
|  | Motor torque (rated/short time S3/S6 25%) |  | N.m | 35.8/95.5 |  |
|  | rotation speed range |  | r/min | 20～12000 |  |
|  | Spindle diameter |  |  | φ150 |  |
|  | Spindle taper |  |  | BT40 |  |
| Guide rail | X-axis guide rail specifications |  |  | 35 |  |
|  | Y-axis guide rail specifications |  |  | 35 |  |
|  | Z-axis guide rail specifications |  |  | 45 |  |
| Drive | Screw specifications X/Y/Z |  | mm | 4020/4012/4012 |  |
|  | Motor power X/Y/Z |  | kw | 3.0/3.0/3.0 |  |
| Speed | Cutting feed speed range |  | mm/min | 1-15000 |  |
|  | Fast-moving speed of X/Y/Z-axis |  | m/min | 30/36/36 |  |
| Machine tool accuracy | Positioning accuracy (X) |  | mm | 0.015 | GB/T18400.4 （Full stroke） |
|  | Repeated positioning accuracy（X） |  | mm | 0.008 |  |
|  | Positioning accuracy (Y/Z) |  |  | 0.01 |  |
|  | Repeated positioning accuracy（Y/Z） |  |  | 0.006 |  |
| Tool magazine（optional） | Tool magazine capacity |  | T | 24 |  |
|  | Tool maximum weight |  | kg | 7 |  |
|  | Tool maximum length |  | mm | 250 |  |
|  | Maximum diameter (full tool /adjacent empty tool) |  | mm | Φ75/Φ150 |  |
| Other | CNC system |  |  | FANUC 0i-MF Plus(5) |  |
|  | Gas source | flow | L/min | ≥280（ANR) |  |
|  |  | air pressure | MPa | 0.6~0.8 |  |
|  | Equipment capacity |  | KVA | 25 |  |
|  | Maximum capacity of water tank |  | L | 400 |  |
|  | Machine tool appearance dimensions |  | mm | 4000*3900（With steps）*2950 | Corresponds to X*Y*Z axis |
|  | Main unit weight (approx.) |  | Kg | 7850 |  |

#: The appearance dimensions of the machine tool may change due to configuration without further notice.

### Standard configuration

| No | Name | Amount | Remark |
| --- | --- | --- | --- |
| 1 | Spindle motor 12000rpm | 1 |  |
| 2 | 24T Disc-type tool magazine | 1 |  |
| 3 | Centralized automatic lubrication system | 1 |  |
| 4 | Explosion-proof work light | 1 |  |
| 5 | Three color warning light | 1 |  |
| 6 | CF card | 1 |  |
| 7 | Fully sealed outer cover sheet metal protection (excluding safety door lock) | 1 |  |
| 8 | Air duct system | 1 |  |
| 9 | Toolboxes and tools | 1 |  |
| 10 | Random set of standard technical documents | 1 | Electronic file |
| 11 | FANUC 0i-MF Plus(5)CNC system | 1 |  |
| 12 | Spindle cooling system | 1 | Water cooling |
| 13 | Rear flushing | 1 |  |

### Customer optional configuration

| No | Name and specification | Selection | Remark |
| --- | --- | --- | --- |
| 1 |  | √ |  |
| 2 |  |  |  |
| 3 |  |  |  |

# Core component brands

| Core component brands and specification |  |  |  |
| --- | --- | --- | --- |
| No | Component name | Brands | Remark |
| 1 | Screw bearing | French FAG/ Japan NSK |  |
| 2 | Spindle unit | Taikan |  |
| 3 | Tool magazine | Taikan |  |
| 4 | Main pneumatic components | SMC/CKD |  |
| 5 | Spindle servo motor | Japan FANUC |  |
| 6 | Drive servo motor | Japan FANUC |  |
| 7 | CNC system | FANUC 0i-MF Plus(5) system |  |
| 8 | Lubrication device | Baotn |  |
| Note: Under the premise of ensuring the quality of the machine tool, in order to ensure the delivery time, the manufacturer of purchased parts of the same grade can be adjusted after consultation. |  |  |  |

# System main function table

（Some functions need to be combined with the configuration of the host to be realized）

| No | Function | clarification | Remark |
| --- | --- | --- | --- |
| 1 | Number of control paths | 1 |  |
| 2 | Maximum total number of control axes (feed axis + spindle) | 6 |  |
|  | Maximum number of feed axes | 5 |  |
|  | Maximum number of spindles | 1 |  |
| 3 | Control the number of axes simultaneously | 4 |  |
| 4 | Display unit | 10.4＂ |  |
| 5 | Servo HRV control | High-speed, high-precision servo control |  |
| 6 | Program capacity | 2M |  |
| 7 | I/O Link | 1024/1024 |  |
| 8 | Automatic data backup | Backup of machine data |  |
| 9 | Memory Card Edit/Run | DNC online processing |  |
| 10 | Program folder management | Functions for using macro programs and machining programs |  |
| 11 | FSSB High-speed rigid tapping | Reduced rigid tapping time and errors |  |
| 12 | Intelligent backlash compensation | Backlash compensation |  |
| 13 | PMC axis control |  |  |
| 14 | Data transfer via embedded Ethernet | Data transfer function via Ethernet |  |
| 15 | USB transfer program to CF card | Data transfer function |  |
| 16 | Spindle Intelligent load meter | Monitoring spindle current and other status |  |
| 17 | Spindle HRV control | High-speed, high-precision servo control |  |
| 18 | AI contour controlⅠ | Pre-reading of program segments within 40 segments (others are not optional) |  |
| 19 | Bell-shaped acceleration and deceleration before read-ahead interpolation |  |  |
| 20 | Intelligent overlap | Achieving reduction of cycle times for machining parts |  |
| 21 | Pre-processing preparation support function |  |  |
| 22 | Imperial/Metric Conversion | Conversion of machine tool units |  |
| 23 | HRV3 control | High-speed, high-precision servo control |  |
| 24 | Flexible feed gear | Arbitrary DMR |  |
| 25 | Minimum setting unit（0.0001mm、0.0001deg、0.00001 inch） | The smallest unit of the system |  |
| 26 | Mirror | Each axis |  |
| 27 | Rigid tapping retraction | Rigid tapping retraction |  |
| 28 | Buffer register | The system can cache data |  |
| 29 | Positioning |  |  |
| 30 | Linear interpolation | Linear machining function |  |
| 31 | Circular arc interpolation | Circular arc processing function |  |
| 32 | Spiral interpolation | Spiral processing function |  |
| 33 | Flip of the rotary axis | Flip of the rotary axis |  |
| 34 | Arbitrary Angle Chamfer/Corner R | Arbitrary Angle Chamfer/Corner R |  |
| 35 | Fixed cycle for drilling | Fixed cycle for drilling |  |
| 36 | No block reference point setting | No block reference point setting |  |
| 37 | Automatic cornering magnification | Automatic corner processing ratio control |  |
| 38 | Scaling | Scaling |  |
| 39 | Plane selection | G17 G18 G19 |  |
| 40 | Programmable mirror | Programmable mirroring of each axis |  |
| 41 | Single program segment | Single-segment of the program can be executed |  |
| 42 | Program restarts | After the program is temporarily interrupted, it can be restarted |  |
| 43 | Tool diameter, tool nose radius compensation | Tool parameter compensation |  |
| 44 | Tool life management | Tool life management |  |
| 45 | Tool life management extension | Tool life management extension |  |
| 46 | Spindle stops accurately |  |  |
| 47 | Position tracking | Position tracking |  |
| 48 | Abnormal load detection | Abnormal load detection for each axis |  |
| 49 | Position switch | Program protection function |  |
| 50 | Backstage Editing | Program editing can be performed in edit mode |  |
| 51 | National language display function | System operation interface language display function 10 kinds |  |
| 52 | Self-diagnosis function | Can monitor machine tool status |  |
| 53 | Help function | Please refer to System Alarm |  |
| 54 | Nano interpolation | Nano interpolation |  |
| 55 | Workpiece coordinate system | G52~G59 |  |
| 56 | Absolute/Incremental Instructions | Can be mixed in the same program section |  |
| 57 | Fault diagnosis | Can find current machine tool fault conditions |  |
| 58 | Clear screen | Manual or automatic clearing of the screen |  |
| 59 | Power consumption monitoring | Can check the power consumption of machine tools |  |

# Recommended oil and grease table

| Used parts | Name | Capacity | Grades | Manufacturer recommendation | Remark |
| --- | --- | --- | --- | --- | --- |
| oil filler | Guide rail oil | 4L | 68# | Shell Tonna T68 or Great wall HG68 | Refill when the oil level is below the minimum level line |
| cutting fluid tank | Water-based cutting fluids | 400L | / | Mobil 242 ，Amer SF19 1SF309 | Add when the liquid level is lower than the minimum liquid level line. Please consult the manufacturer for specific usage. |
| cutting fluid tank | Oil-based cutting fluids | 400L | / | Div DW-812，Amer C13B/ LX-C13 | Add when the liquid level is lower than the minimum liquid level line. Please consult the manufacturer for specific usage. |
| Water cooler | coolant | 15L | G48 | BASF/G48/green | Replace it after 3 months of initial use |
| Requirements: For the cutting fluid in use, please follow the manufacturer's instructions to properly manage the pH, the mixing degree of the original solution and dilution water, the salt concentration of the dilution water, and the frequency of cutting fluid replacement. If you use cutting fluids and guide oils other than those recommended, pay full attention to the usage methods and conditions. Depending on the interaction of the coolant and guide oil, etc., it may sometimes cause poor lubrication of the screw and guide rails and the generation of scum, which will reduce their durability. Therefore, please check with the manufacturer carefully before use. |  |  |  |  |  |

# Routine maintenance and upkeep of machine tools

Please follow the "Operating Instructions" for daily, weekly, monthly, quarterly and annual inspections and maintenance; the disassembled parts must be reinstalled after completion of maintenance.

# Quality assurance

- The quality guarantee period is 12 months from the date of final acceptance.

- During the warranty period, if the buyer uses the equipment or machine tool under normal conditions in accordance with the instruction manual, the seller will be responsible for any damage caused by defects in factory design or manufacturing. During the above warranty period, if there is a problem with the equipment, the seller will make correction suggestions to the buyer; if the buyer still cannot solve the problem according to the seller's instructions and system description, the seller shall dispatch a maintenance engineer free of charge within 2 working days after receiving the buyer's notice. Arrive at the buyer's factory.

- During the warranty period, the seller shall provide the buyer with timely maintenance services and necessary spare parts free of charge. It is verified that the failure loss caused by the buyer's improper operation, unreasonable cutting parameter selection or self-disassembly and modification, and other non-manufacturing quality reasons, as well as the damage to the appearance or interior of the machine tool caused by excessive storage time, are not covered by the above warranty, and the buyer must pay for relevant maintenance cost.

- After the warranty period, the seller will maintain the machine tool for life and provide free online services and technical guidance. Provide timely supply of spare parts to buyers at favorable prices. If it is necessary to send someone to the site for repairs, the corresponding fees will be charged according to the seller's standards.

# Others

1. Matters not covered in this agreement shall be resolved through negotiation between the two parties.

2. This agreement is an attachment to the contract and will take effect simultaneously with the contract after it is signed by Party A and Party B.

3. This agreement is made in duplicate, with Party A and Party B each holding one copy.